Press release

A dynamic exhaust air purification concept from Dürr CTS instead of continuous full-load operation

Marabu cuts gas consumption by over 50% with load-dependent air pollution control

Bietigheim-Bissingen, Germany, March 25, 2026 – The printing ink manufacturer Marabu has completely modernized its emission treatment system at its Tamm, Germany facility. The system, implemented by Dürr CTS, combines concentration and recuperative thermal oxidation with a multi-stage, load-dependent control strategy. In January, the system operated in optimized mode approximately 80 percent of the time, reducing gas consumption by more than half compared to conventional continuous operation.

The paint and printing ink manufacturer Marabu is currently modernizing its production facilities with a focus on sustainability, efficiency, and productivity. Part of this effort involves upgrading its air pollution control system. Many medium-sized paint and coating manufacturers in Europe, especially in Germany, installed emission treatment systems after the first German Technical Instructions on Air Quality Control (TA Luft) came into effect in the 1990s. These systems are now nearing the end of their service life. Based on the state of the art at the time, they were designed for the maximum expected pollutant load in the production exhaust air. Since then, even though paints and coatings are now manufactured with significantly lower solvent emissions, the systems have been continued running at full capacity, with correspondingly high energy consumption.

Marabu manufactures printing inks and liquid coatings for industrial applications. The expansion and modernization of its production facilities were expected to increase the volume of exhaust air by a quarter, to up to 20,000 standard m³ per hour. Marabu therefore opted for a new plant design from Dürr CTS that ensures compliance with emissions regulations, increases capacity, and significantly reduces energy costs.

The unique feature lies in the operating mode

The technical design of the new system is well established in the industry: concentration followed by recuperative thermal oxidation. The Sorpt.X CD rotary adsorber concentrates the solvents into a reduced flow rate, which is then fed into the Oxi.X TR recuperative thermal oxidizer. A downstream heat exchanger typically uses the process heat to preheat the small volume of desorption air. The unique feature of the new system lies in its operating mode. “We no longer generally operate at full load but instead adjust the thermal output to the actual pollutant load. To do this, sensors continuously measure the actual solvent concentration. This intelligent control system offers enormous potential for long-term energy and operating cost savings,” explains Ralf Brenner, Sales Manager at Dürr CTS.

Depending on the measured solvent concentration, the system switches between normal and optimized operation. Initial operating data confirm the concept. In January, due to low solvent emissions from paint and varnish production, the system operated in optimized mode approximately 81 percent of the time; this translates to a 51.6 percent reduction in gas consumption compared to static continuous operation. “The initial evaluations show that performance, cost-effectiveness, and environmental protection can be combined,” says Jürgen Schwarz, Plant Manager for Tamm and Bietigheim-Bissingen at Marabu. “We meet the emission requirements, have no odor nuisance, and our energy consumption has dropped significantly.”

Safety through process precision

Operating an air pollution control system with a concentration unit in a dynamic manner requires extensive process engineering expertise and experience in managing the time-delayed reactions of the adsorber and thermal oxidation. To ensure safe operation of the system under varying solvent concentrations, Dürr CTS analyzed the process, conducted a thorough risk assessment, and defined clear switching criteria that guarantee emission safety and explosion protection, even during load changes. Continuous LEL measurement monitors actual pollutant levels, while frequency-controlled fans regulate volume flows across operating modes. In normal operation, the system operates with a higher desorption air volume to ensure stable oxidation conditions, even under increasing load. At low solvent concentrations, the system switches to optimized operation with approximately 60 percent less desorption air. As a result, the thermal oxidation system heats a significantly smaller volume flow, reducing gas consumption accordingly.

For Marabu, the new exhaust air treatment system is a key component of its production modernization. It significantly reduces emissions and energy consumption and demonstrates that the modernization of an existing plant can be implemented in a way that makes both ecological and economic sense.

Image 1: Air pollution control system at the printing ink manufacturer Marabu: on the left, the recuperative thermal oxidizer; on the right, the concentration unit.

Image 2: Commissioning of the thermal oxidation system by Dürr CTS specialist personnel.

About Dürr CTS

Dürr CTS (Clean Technology Systems) is a global leader in environmental technology for industrial applications. The company designs and delivers customized solutions for air pollution control, noise abatement, and decentralized power generation from waste heat.

With approximately 1,300 employees across 12 countries, Dürr CTS supports customers worldwide in reducing emissions, improving energy efficiency, and making production more sustainable.

With more than 60 years of experience in industrial environmental technology, Dürr CTS was formerly part of the Dürr Group. In 2024, Dürr CTS generated annual revenue of more than 400 million euros. Since November 2025, the company has operated independently. Ownership is divided between Stellex Capital Management (75 percent) and the Dürr Group (25 percent).

Contact

# Dürr CTS GmbH

Global Marketing

E-mail: info@cts-durr.com

www.cts-durr.com